# Falcon-8 CPU Documentation

Author: Abdullah Almutairi
Processor: Falcon-8
Version: v1.0
Architecture: Single-Cycle 8-bit Processor

## 1. CPU Specifications

| Item | Value |
| --- | --- |
| CPU Name | Falcon-8 |
| Version | v1.0 |
| Architecture | Single-Cycle |
| Data Width | 8-bit |
| Instruction Width | 16-bit |
| Program Counter | 8-bit |
| Registers | 8 General Purpose Registers |
| Register Width | 8-bit |
| Instruction Memory | ROM 256 x 16-bit |
| Data Memory | RAM 256 x 8-bit |
| Address Space | 256 Addresses |
| Flags | Zero, Carry |

## 2. Instruction Format

Instruction Layout:
[15:11] Opcode (5 bits)
[10:8]  RD
[7:5]   RS
[4:0]   Immediate

## Register Encoding

| Register | Binary |
| --- | --- |
| R0 | 000 |
| R1 | 001 |
| R2 | 010 |
| R3 | 011 |
| R4 | 100 |
| R5 | 101 |
| R6 | 110 |
| R7 | 111 |

## Instruction Set

| Mnemonic | Opcode | Description |
| --- | --- | --- |
| ADD | 00000 | RD = RD + RS |
| SUB | 00001 | RD = RD - RS |
| AND | 00010 | RD = RD AND RS |
| OR | 00011 | RD = RD OR RS |
| XOR | 00100 | RD = RD XOR RS |
| NOT | 00101 | RD = NOT RD |
| LOAD | 00110 | Load RAM[Imm] into RD |
| STORE | 00111 | Store RD into RAM[Imm] |
| MOV | 01000 | RD = RS |
| LDI | 01001 | Load Immediate into RD |
| JMP | 01010 | Jump to Immediate |
| JZ | 01011 | Jump if Zero |
| JNZ | 01100 | Jump if Not Zero |
| JC | 01101 | Jump if Carry |
| HLT | 01110 | Stop execution |

## 3. Architecture

Main Components:
• Program Counter
• Instruction ROM
• Control Unit
• Register File
• Arithmetic Logic Unit (ALU)
• Data RAM
• Write-Back Unit

Execution Flow:
PC -> ROM -> Control Unit -> Register File -> ALU -> RAM (optional) -> Write Back -> Register File

## 4. Engineering Log

### Bug #1

Register address mapping corrected: Address_A = RS, Address_B = RD, WriteAddr = RD.

### Bug #2

RegWrite remained enabled after execution. Fixed through Control Unit.

### Bug #3

MemToReg signal direction corrected.

### Bug #4

RAM write logic restricted to STORE instruction.

### Bug #5

Branch logic corrected for JMP/JZ/JNZ/JC.

### Bug #6

Empty ROM locations (0000) interpreted as ADD. Programs now terminate using HLT.

### Bug #7

Verified each module independently before full CPU integration.

## Lessons Learned

This project covered:
• CPU Architecture
• Instruction Set Architecture (ISA)
• Register File Design
• ALU Design
• Datapath Design
• Control Unit Design
• Memory Interface
• Hardware Debugging
• Processor Verification

## 5. Project Summary

Architecture

Single-Cycle 8-bit CPU

Modules Implemented

- Program Counter

- ROM

- Control Unit

- Register File

- ALU

- RAM

- Branch Unit

- Write Back

Supported Instructions

15 Instructions

Verification

All instructions have been functionally verified using custom assembly test programs.

Future Version

Falcon-X Processor (Under Planning)